| ECE356 Midterm |  |
| --- | --- |

- (10 points) The following two problems are related to the Amdahl’s law:

- What does Amdahl’s Law state? (In words, not a formula.)

- Assume that 40% of the runtime of a certain application cannot be parallelized. What is the maximum possible speedup by parallelization for this application if we assume that the input set is fixed?

- Suppose that after an application has been made into parallel by running on N processors, it achieves a speedup of 5. What is the possible range, in terms of percentage, for the serial part of the original application? (Serial means it cannot be made into parallel).

Solution: Amdahl’s Law states the relationship on the percentage of a program that we can execute in parallel to the overall speedup of the entire program. Amdahl's Law argues that the overall speedup of a system is limited by the portion of the system that cannot be improved or parallelized.

For part b, the maximum speedup is 2.5 times as the non-parallel part is 40%.

For part c, observe that N must be larger than 5 to achieve a speedup of 5, if there is a serial part. That is, if serial part is 0, then N can be 5. If N is 6, we can calculate the serial part is 0.04. If N is infinity, then x is 0.2. Therefore the range of possible percentage of serial part is [0, 0.2)

- (10 points) Two’s complement for integers

a)   How to represent negative numbers in two’s complement format?

b)   For a 4-bit two’s complement number, show how to represent the following operations and their results. If you cannot, explain why.

1) 4-7

2) 12-2

Solution: refer to the textbook section 2.4. Two's complement is a method for representing positive and negative integers in binary form. It allows the use of binary subtraction methods and simplifies the design of digital systems.

Note that for part 2, the 12 cannot be represented with 4-bit two’s complement format. Hence, there is an overflow.

- (10 points) True or false: decide each of the following statements as true or false, and give brief explanations for the false ones.

- With future hardware advances, according to the Moore’s law, the clock frequency of CPUs will double every 18-24 months.

- For JAVA virtual machines, one big advantage is that the compiled virtual machine instruction code can run on different hardware platforms.

- The IEEE 754 floating point standard only allows finite number of floating point representations

- The cache memory is part of the DRAM memory on the CPU.

- To compare the performance of CPUs, the one with lower CPI will have better performance.

Solution: only b, c are correct, others are false.

For a, Moore’s law is about the number of transistors not frequency.

For d, cache is implemented with SRAM, not DRAM.

For e, we should not compare CPU performance with CPI.

- (15 points) A company has developed a processor that runs at 2.5GHz. To test this processor, the company runs a benchmark program that is compiled into the following four classes of instructions with their frequency and clock cycles as follows:

| Class | Frequency | Clock Cycles |
| --- | --- | --- |
| A | 20% | 1 |
| B | 40% | 2 |
| C | 20% | 3 |
| D | 20% | 5 |

- What is the average CPI of the benchmark program?

- Suppose that the compiler has options to change the distributions of class A, class B, and class D instructions simultaneously.  If we reduce the class D instructions by half (e.g., ten instructions reduced to five), suppose that class A and class B instructions always increase by the same percentage, by what is the maximum percentage of increase that class A and B instructions can have without causing an increase in the overall execution time of this program?

Solution: First we can calculate the CPI as 20% x 1 + 40% x 2 + 20% x 3 + 20% x 5 = 2.6.

Further, we know that CPI x NumberOfInstructions = ClockFrequency x Execution Time, For part b, we can increase the frequency of class A instructions and B to ensure the overall execution time is the same, therefore we have (by using class A/B instructions) for using the ratio of the new class A instructions to old class A instructions as X:

X*0.2+X*0.8+0.6+1.0/2 = 2.6, therefore, X = 1.5, and this corresponds to an increase in

percentage to be 50 percent.

5. (15 points) Write assembly for the algorithm below.

c = a + b [3];

do {

c -= a;

} while (c > 3)

Suppose that a is in X10, b base address is in X11, c is in X12. All variables are words,

which is eight-byte long (64-bit processor). Write comments for your code.  Please use

registers as you need.

Refer to chapter 2 for branches and assembly code. There is no single correct answer. Partial credit will be given based on the solutions.

6 (10 points)

According to the IEEE 754 floating point standard, the density of such representations may not be evenly

distributed. Consider a region of (0,2) and (100,102). Which region do you think contains more

representations of floating points? Which region has more rounding errors in calculations? Explain your

logic..

(0,2) will be more dense as the scientific notation needs to represent more numbers in the range near 0.  For the similar reason, the region of (100,102) has more rounding errors.

7. (20 points)

Here is a series of address references given as word addresses: 4, 12, 8, 6, 24, 26, 98, 41, 11, 1, 3, 14, 26, and 116.

Part a: Show the hits and misses and final cache contents for a direct-mapped cache with one-word blocks and a total size of 8 words.

Part b: Show the hits and misses and final cache contents for a four-way set-associative cache with two-word blocks and a total size of 16 words. Assume LRU replacement.

Write down your calculation steps and the final result.

Part c: Consider a two-level cache hierarchy as follows. Assume initially all caches are empty. If there is a

miss in L1 or L2, the next level of the memory is accessed until the data is located. If both L1 and L2 are

misses, then the data is read into both levels.

L1: Direct-mapped cache with one-word blocks and a total size of 8 words

L2: Fully set-associative cache with four-word blocks and a total size of 32 words (using LRU replacement)

For the same address sequence, trace all accesses showing L1 and L2 hits/misses.

Calculate the global miss rate (misses to main memory / total accesses)

Calculate the average memory access time (AMAT) for each cache configuration if L1 hit takes 1 cycle,

L2 hit takes 10 cycles, and main memory access takes 100 cycles.

Write down your calculation steps and the final result.

a)

In a direct-mapped cache, each memory location is mapped to exactly one cache line. The cache line is determined by (Memory Address) MOD (Number of Cache Lines).

For example, address 4 maps to cache line 4 MOD 8 = 4. So the first access is a miss.

It is easy to determine that all accesses are misses.

The final state of the cache is as follows, where 0:24 means the line 0 now contains the block for address 24.

0:24, 1:1, 2:26, 3:3, 4:116, 6:14

b)

4(miss), 12(miss), 8(miss), 6(miss), 24(miss), 26(miss), 98(miss), 41(miss), 11(miss), 1(miss), 3(miss), 14(miss), 26(hit), and 116(miss).

| Set 0 | Set 0 | Set 0 | Set 0 | Set 0 | Set 0 | Set 0 | Set 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 116 | 12 | 8 | 6 | 24 | 26 | 98 | 14 |
| Set 1 | Set 1 | Set 1 | Set 1 | Set 1 | Set 1 | Set 1 | Set 1 |
| 11 | 1 | 3 |  |  |  |  |  |

c) Depending on the exact details of the two level cache the results may be slightly different.

Global miss rate = # of misses to main memory / total references = 8/14  or 57.14%

Average Memory Access Time (AMAT)

- L1 hit time = 1 cycle, but we have 0% L1 hits, so effectively every reference is an L1 miss.

- On an L1 miss:

  - If L2 hits, the total cost = 1 (to check L1) + 10 (L2 hit) = 11 cycles.

  - If L2 misses, total = 1 + 10 + 100 = 111 cycles.

- By emulating the hits and misses, we have 6 L2 hits and 8 L2 misses, total 14:

Total cycles  =  6×11  +  8×111  =     954

AMAT = 954/14 or 68.14 cycles

Another way of getting AMAT is through the standard formula as:

AMAT = 1+1 [ 10 x 6/14 + 100 x 8/14 ] = 68.14 cycles

8. Suppose that you want to achieve a speed-up of M with N processors in a multiprocessor system.

According to the Amadahl’s law, what is percentage, P, of the original computation, can be still

sequential in the whole program? Now set N = 100, draw a graph showing the relation of M and P in a

graph (you may want to calculate 10-12 points of M and P pairs to draw this graph accurately). Finally,

set M = 50, can you draw a graph showing the relation of N and P? (You may notice that for this M, some

N values are not possible. You do not need to plot these impossible values in the graph)

M  = 1 / (P + (1-P)/N) Hence, the figure can be drawn for M and P is N is set as 100, similarly, the figure can be drawn for P and N if M=50.